Overview:

During the 2024 stimulations there were three wells equipped with downhole instruments for microseismic monitoring: 58-32, 78B-32 and 56-32.

Initial planing:

Well 58-32: 12-level geochain (SLB)

Well 78B-32: 8-level geochain (GES)

Well 56-32: Single-level PSS tool (GES)

Deviations:

Well 58-32:

→ 12-level geochain pulled after 3 days of recording (2024-04-06)

→ missing data in between, SLB report pending

→ replaced by DAS wireline on 2024-04-17 (not included in this data set)

Well 78B-32:

→ 8-level geochain failed on 2024-04-18, pulled on 2024-04-19

→ geochain replaced by single PSS tool on 2024-04-20 ~00:00

(at level 1 position in raw data)

→ missing/empty data in between (see log below)

→ receiver 8 written to receiver 7 position in between (see log below)

Well 56-32:

→ Single-level PSS tool failed/replaced between 2024-04-05 and 2024-04-07

Fig 1: Downhole data availability for the 2024 stimulations at Utah FORGE. The data availability shown here does not necessarily guaranty uncorrupted data (see failure log below)

Data delivery:

raw

→ segy/seg2 files as delivered by contractors/partners

mseeds

→ mini-seed files with metadata matching the provided receiver files

→ naming convention: increasing receiver index from top to botttom

→ horizontal components have arbitrary orientations, N and E components not rotated

metadata

→ additional data files such as logs, coordinates, etc.

Reference of local coordinate system:

well 16A, ground level (lat = 38.50402147, lon=-112.8963897, elevation=1650 m)

The receiver coordinates given in the receiver_ft.csv and receiver_meter.csv are relatibe to the same reference. Deviating elevations/depths given in FORGE16AandB_StimulationSensors.xlsx might have other local references, e.g., the ground level at the given well.

Time:

All seismic traces are in UTC.

Field notes (Outages and possible timing errors) for wells 56-32 and 78B-32:

→ from GES field notes (local time) → here converted to UTC

→ full log in local time in spreadsheet delivered with the data

2024-03-31

15:30	Clock time on the PSS made an error switching due to a summertime clock change.

Merged PSS and Geochain files 07:00-15:30  likely to be mis-matched by an hour

this is not worth correcting as we are only testing at the moment

2024-04-05

20:19	Around this time the PSS in 56 failed.

2024-04-07:

16:00	Replaced the PSS tool

17:40	56 is detecting events again

2024-04-10

15:00	Geochain has failed overnight. Restart around 15:30

23:10	Geochain off to investigate the clock problem

23:35	Geochain on

2024-04-11

00:02	Geochain off. It failed for an unknown reason but manually re-started OK

00:18	Geochain On.

22:14	Re-clamped groups 1, 5 and 8 of 78B

2024-04-12

22:45	Geochain ping test. All channels are normal

2024-04-14

20:27	Geochain tool 7 failed. Re-started but is noisy

Various fails and re-starts

Discovered that the tool 8 data is written to traces 19, 20 and 21 when tool 7 fails 		(this is referred to as "bypass" in Avalon software)

23:39	Tool 7 is back on.

2024-04-15

Water dripping slightly from 78B

2024-04-18

08:40	Geochain in 78B failed overnight. Reported by Ben K. from the UK

Merger stopped due to missing Geochain files

Re-triggered the data with only one group and two traces required ie the PSS

2024-04-19

01:00	Finished pulling the Geochain

2024-04-21

15:00	Tools are running

2024-04-25

15:00	Pulling 78B. Turned it off the trigger

Instrument information for wells 56-32 and 78B-32 (GES):

→ see spreadsheet delivered with the data for response infromation

→ trace amplitudes are positive for energy propagating in the directions up, +1, +2, 			with 1 and 2 being the arbitrary orientations of the two horizontal channels

→ sampling rate 4000Hz

Field notes (Outages and possible timing errors) for well 58-32 (SLB):

→ Data from well 58-32 (SLB) were not integrated in real-time downhole monitoring

→ The timing was not synchronized with other deployments

→ There are timing errors between geophone groups

→ final report of SLB still pending

Instrument information for wells 58-32 (SLB):

→ sampling rate 2000Hz

→ SLB report pending

Data usage from 58-32 (SLB) only recommended after extensive quality control!